# Macro detectPigment_quantifySignalAround_vf.ijm

The purpose of this macro is to detect pigments (seen as black spots) on a transmitted light image and to analyse signal in other channels around those pigments. The pigments can also be present in another fluorescent channel.

Careful, this macro is compatible with Fiji 1.52p and not later versions! In following versions, the detected pigments are not located at the correct slice. We think (but did not check) that this could be corrected by using ROI.setPosition(slice) for each ROI, function that was added in version 1.53b.

If the version is not 1.52p, the macro stops with this error:

## A/ Steps of the macro:

1/ The macro asks for the image to analyse:

This stack image should have at least a transmitted light channel. On this channel, the pigments are detected as the “black spots”. In the following they may be called “black” pigments.

2/ The macro makes a pause so that the user can look at the slices to keep:

The user should check which slices contain significant (not out of focus) signal and click OK when done.

3/ The user must fill parameters from acquisition and for analysis; this is a very important step to obtain a correct detection:

Slices to keep for analysis (remove out of focus slices for a more precise analysis)

Accepted distance between black and red pigments to be considered the same (distance due to axial chromatic aberration)

Parameter for detection of pigments on the red image: Find Maxima parameter (see below for explanations)

Proportion of positive (i.e. above a threshold) pixel in red channel to consider a black pigment red

Parameters for detection of pigments on the transmitted light image:

- Find Maxima parameter (see below for more explanations)

- Threshold: we keep spots with minimum < to the value (-1: we keep all the spots)

Pixel size of the image & step in Z (both in µm)

Spheroid size for analysis of the intensity in other channel(s) around the pigment, in (X,Y,Z)

Ex:  with a pixel size of 0.1µm, a Z-step of 0.2µm:

- If  “For the connexion between red and trans spots” the values are in (X,Y) 0.2µm (= 2 pixels) and in Z 0.4µm (=2 slices), for a black pigment detected in pixel XBP,YBP,ZBP, a red pigment  located in pixel XRP,YRP,ZRP such as:

XBP-2≤XRP≤ XBP+2YBP-2≤YRP≤ YBP+2ZBP-2≤ZRP≤ ZBP+2 is considered the  same pigment.

- If “For the analysis of proteins around the pigment” the values are in (X,Y) 0.5µm (= 5 pixels) and in Z 0.4µm (=2 slices), all intensity analysis (final result table) will be done within a spheroid, centred on the pigment detected by the FindMaxima and of radius 5 pixels in (X,Y) and on 5 slices: the one of the pigment + 2 above + 2 below

4/ The macro asks the channels correspondence and as an option if pigments should also be detected on the red channel:

For any other channel than the “Cyan (nucleus)”, signal will be analysed around the pigment and results will appear in the final Results table.

5/ Draw all the ROIs of study (they must be added in the Manager to be treated):

The code continues if at least one ROI has been drawn (it keeps asking you to draw one otherwise).

6/ For each ROI drawn in step 5:

a/ Pigment detection is performed on the transmitted light image (thanks to a FindMaxima on the Minimum z-projection of the transmitted light channel, pigments are detected-as black spots- and are kept only the ones below the threshold specified in the interface of step 3/). The user can (manually) remove some of the pigments:

Be careful not to remove the ROIs cell.

b/ [Optional] If in step 4/ the “Detect pigments on red channel (if exists)” was checked and one channel was specified as “Red”, the same appears for the red channel; the names of the spots contain trans and/or red depending on the channel on which it was detected:

Due to axial chromatic aberration the same pigment in red and transmitted light can be a bit shifted; for this reason, the macro “associates” (i.e. considers as the same) 2 spots (trans & red) if they are located in a spheroid centred on the black spot; the radius in X,Y and number of slices in Z are chosen in the interface in the parameters “For the connection between red and trans spots”:

c/ For the channels specified red/green/blue chosen in interface 4/, a threshold is applied on the maximum-z projection (automatic Triangle method), and used on the stack to define the presence or absence of the corresponding proteins:

7/ The result table recapitulates for each channel analysed in 6/b/:

- Max and mean values are calculated on the spheroid defines in the interface of step 3/ in the part “For the analysis of proteins around the pigment”

- Positiveness for one channel is computed as yes if the maximum in the spheroid is above the threshold computed in step 6/c/

This result table as well as the ROIs (cell(s) + pigments) are saved in the folder containing the data.

How to choose the find maxima parameters?

Transmitted light channel:

- Duplicate the stack for the channel of transmitted light channel, for the slices you will use for the analysis

- Perform the “Minimum Intensity” Z-projection (Image > Stacks > Z project…)

- Adjust Brightness & Contrast if necessary (this Option will be blocked once you launched FindMaxima)

- Process> Find Maxima on this image. Check the best parameter for “Prominence” (use the “Preview point selection” check); be careful to check “Light Background”; this best “Prominence” value is the one you should enter in step 3/

Red:

- Duplicate the stack for the channel red channel, for the slices you will use for the analysis

- Perform the “Maximum Intensity” Z-projection (Image > Stacks > Z project…)

- Adjust Brightness & Contrast if necessary (this Option will be blocked once you launched FindMaxima)

- Process> Find Maxima on this image. Check the best parameter for “Prominence” (use the “Preview point selection” check); be careful to un-check “Light Background”; this best “Prominence” value is the one you should enter in step 3/